EXPERIMENT NO.1

- VHDL CODE:

-- Experiment No 1. To Study Design Flow using an EDA Tool

-- Name : Ritesh Machindra Todmal

-- Roll No. : 404C001

-- Teacher Name: Dr.D.G.Ganage

-- Batch : C1

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity FA is

Port ( A : in  STD_LOGIC;

B : in  STD_LOGIC;

Cin : in  STD_LOGIC;

Sum : out  STD_LOGIC;

Cout : out  STD_LOGIC);

end FA;

architecture Behavioral of FA is

begin

Sum <= (A xor B xor Cin);

Cout <= ((A and B) or (B and Cin) or (A and Cin));

end Behavioral;

- VHDL TESTBENCH:

-- Experiment No 1- To Study Design Flow using an EDA Tool

-- Name : Ritesh Machindra Todmal

-- Roll No. : 404C001

-- Teacher Name: Dr.D.G.Ganage

-- Batch : C1

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

ENTITY FA_Test IS

END FA_Test;

ARCHITECTURE behavior OF FA_Test IS

COMPONENT FA

PORT(

A : IN  std_logic;

B : IN  std_logic;

Cin : IN  std_logic;

Sum : OUT  std_logic;

Cout : OUT  std_logic

);

END COMPONENT;

--Inputs

signal A : std_logic := '0';

signal B : std_logic := '0';

signal Cin : std_logic := '0';

--Outputs

signal Sum : std_logic;

signal Cout : std_logic;

-- No clocks detected in port list. Replace <clock> below with

-- appropriate port name

BEGIN

-- Instantiate the Unit Under Test (UUT)

uut: FA PORT MAP (

A => A,

B => B,

Cin => Cin,

Sum => Sum,

Cout => Cout

);

-- Stimulus process

stim_proc: process

begin

a <= '0';

b <= '0';

Cin <= '0';

wait for 20 ns;

a <= '0';

b <= '1';

Cin <= '0';

wait for 20 ns;

a <= '1';

b <= '0';

Cin <= '1';

wait for 20 ns;

a <= '1';

b <= '1';

Cin <= '1';

wait for 20 ns;

end process;

END;

- SIMULATION RESULT

- SCHEMATICS (RTL Technology)

=========================================================================

*	Final Report	*

=========================================================================

Final Results

RTL Top Level Output File Name	: fa.ngr Top Level Output File Name	: fa

Output Format	: NGC

Optimization Goal	: Speed

Keep Hierarchy	: No

Design Statistics

# IOs	: 5

Cell Usage :

# BELS	: 2

#	LUT3	: 2

# IO Buffers	: 5

#	IBUF	: 3

#	OBUF	: 2

=========================================================================

Device utilization summary:

Selected Device : 3s250epq208-5

| Number | of | Slices: | 1 | out | of | 2448 | 0% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number | of | 4 input LUTs: | 2 | out | of | 4896 | 0% |
| Number | of | IOs: | 5 |  |  |  |  |
| Number | of | bonded IOBs: | 5 | out | of | 158 | 3% |