EXPERIMENT NO.5

- VHDL CODE:

-- Experiment No 5

-- Name : Ritesh Machindra Todmal

-- Roll No. : 404C001

-- Teacher Name: Dr.D.G.Ganage

-- Batch : C1

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_unsigned.ALL; use IEEE.STD_LOGIC_arith.ALL;

entity FIFO_4bit is

Port ( clk,rst : in STD_LOGIC; enr,enw : in STD_LOGIC;

datain : in STD_LOGIC_VECTOR (3 downto 0); dataout : out STD_LOGIC_VECTOR (3 downto 0); full,empty : out STD_LOGIC);

end FIFO_4bit;

architecture Behavioral of FIFO_4bit is

type memory_type is array (0 to 7) of std_logic_vector(3 downto 0); signal memory : memory_type;

signal readptr,writeptr : std_logic_vector(3 downto 0):="0000";

--signal cnt:std_logic_vector(24 downto 0);

--signal clk1:std_logic;

begin

FIFO_Process: process(clk)

begin

if(clk'event and clk='1' and enw ='1' and enr='0') then --WRITEOPERATION

memory(conv_integer(writeptr)) <= datain; writeptr <= writeptr + '1';

end if;

if(clk'event and clk='1' and enr ='1' and enw='0') then --READ

OPERATION

dataout <= memory(conv_integer(readptr));

readptr <= readptr + '1';

end if;

if(readptr = "1000") then

empty<='1';

else

empty<='0';

end if;

if(writeptr = "1000") then

full <='1';

else

full <='0';

end if; end process;

end Behavioral;

- VHDL TESTBENCH:

-- Experiment No 5

-- Name : Ritesh Machindra Todmal

-- Roll No. : 404C001

-- Teacher Name: Dr.D.G.Ganage

-- Batch : C1

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

-- Uncomment the following library declaration if using

-- arithmetic functions with Signed or Unsigned values

--USE ieee.numeric_std.ALL;

ENTITY test1 IS END test1;

ARCHITECTURE behavior OF test1 IS

-- Component Declaration for the Unit Under Test (UUT) COMPONENT FIFO_4bit

PORT(

clk : IN std_logic; rst : IN std_logic; enr : IN std_logic; enw : IN std_logic;

datain : IN std_logic_vector(3 downto 0); dataout : OUT std_logic_vector(3 downto 0); full : OUT std_logic;

empty : OUT std_logic

);

END COMPONENT;

--Inputs

signal clk : std_logic := '0'; signal rst : std_logic := '0'; signal enr : std_logic := '0'; signal enw : std_logic := '0';

signal datain : std_logic_vector(3 downto 0) := (others => '0');

--Outputs

signal dataout : std_logic_vector(3 downto 0); signal full : std_logic;

signal empty : std_logic;

-- Clock period definitions

constant clk_period : time := 10 ns; BEGIN

-- Instantiate the Unit Under Test (UUT) uut: FIFO_4bit PORT MAP (

clk => clk, rst => rst, enr => enr,

enw => enw, datain => datain,

dataout => dataout, full => full,

empty => empty

);

-- Clock process definitions clk_process :process

begin

end process;

clk <= '1'; wait for 5 ns; clk <= '0'; wait for 5 ns;

-- Stimulus process stim_proc: process begin

enw<='1';

enr<='0'; datain<="0001"; wait for 10ns;

datain<="0011"; wait for 10ns;

datain<="0111"; wait for 10ns;

datain<="1111"; wait for 10ns;

datain<="1110"; wait for 10ns;

datain<="1100"; wait for 10ns;

datain<="1000"; wait for 10ns;

datain<="0000"; wait for 10ns;

enw<='0';

enr<='1';

wait for 80ns;

end process; END;

- SIMULATION RESULT

- SCHEMATICS (RTL Technology)

=========================================================================

*              Final Report             *

=========================================================================

Final Results

RTL Top Level Output File Name	: ALU_4bit.ngr Top Level Output File Name	: ALU_4bit

Output Format	: NGC

Optimization Goal	: Speed

Keep Hierarchy	: No

Design Statistics

# IOs	: 15

Cell Usage :

# BELS	: 30

#	INV	: 1

#	LUT2	: 2

#	LUT3	: 2

#	LUT4	: 18

#	MUXF5	: 7

# IO Buffers	: 15

#	IBUF	: 11

#	OBUF	: 4

=========================================================================

Device utilization summary:

Selected Device : 3s250epq208-5

| Number | of | Slices: | 13 | out | of | 2448 | 0% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number | of | 4 input LUTs: | 23 | out | of | 4896 | 0% |
| Number | of | IOs: | 15 |  |  |  |  |
| Number | of | bonded IOBs: | 15 | out | of | 158 | 9% |

Partition Resource Summary:

No Partitions were found in this design.

=========================================================================