EXPERIMENT NO.6

VHDL CODE

library IEEE;

use IEEE.STD_LOGIC_1164.ALL; use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity lcd is

Port ( clk,reset : in std_logic; RS,EN : out std_logic;

data : out std_logic_vector(7 downto 0));

end lcd;

architecture Behavioral of lcd is

type state_type is (s0,s1,s2,s3,s4,s5,s6,s7,s8,s9,s10,s11, s12,s13,s14,s15,s16,s17,s18,s19,s20,s21,s22,s23);

signal state:state_type; begin

process(clk, reset, state) begin

if reset = '1' then

state <= s0;

elsif rising_edge(clk) then if state = s0 then

state <= s1;

RS<='0'; -- Write commands to LCD. EN <= '1';

data <= "00110000"; -- Function set for 8 bit interface, 1 line mode and 5x7 dot matrix.

end if;

if state = s1 then

state <= s2; EN <= '0';

ON.

end if;

if state = s2 then

state <= s3; EN <= '1';

data <= "00001111"; -- Display cursor and blinking

end if;

if state = s3 then

state <= s4; EN <= '0';

end if;

if state = s4 then

state <= s5; EN <= '1';

data <= "00000001";	-- Clear display.

end if;

if state = s5 then

state <= s6; EN <= '0';

end if;

if state = s6 then

end if;

state <= s7; EN <= '1';

data <= "10000100"; -- Display address.

if state = s7 then

state <= s8; EN <= '0';

end if;

if state = s8 then

RS <= '1'; -- Write data to LCD. state <= s9;

EN <= '1';

data <= "01010111"; --W

end if;

if state = s9 then

state <= s10; EN <= '0';

end if;

if state = s10 then

state <= s11; EN <= '1';

data <= "01000101"; --E

end if;

if state = s11 then

state <= s12; EN <= '0';

end if;

if state = s12 then

state <= s13; EN <= '1';

data <= "01001100"; --L

end if;

if state = s13 then

state <= s14; EN <= '0';

end if;

if state = s14 then

state <= s15; EN <= '1';

data <= "10110000"; --(-)

end if;

if state = s15 then

state <= s16; EN <= '0';

end if;

if state = s16 then

state <= s17; EN <= '1';

data <= "01000011"; --C

end if;

if state = s17 then

state <= s18; EN <= '0';

end if;

if state = s18 then

state <= s19; EN <= '1';

data <= "01001111"; --O

end if;

if state = s19 then

state <= s20; EN <= '0';

end if;

if state = s20 then

state <= s21; EN <= '1';

data <= "01001101"; --M

end if;

if state = s21 then

state <= s22; EN <= '0';

end if;

if state = s22 then

state <= s23; EN <= '1';

data <= "01000101"; --E

end if;

if state = s23 then

EN <= '0';

end if;

end if; end process;

end Behavioral;

=========================================================================

*	Final Report	*

=========================================================================

Final Results

RTL Top Level Output File Name	: lcd.ngr Top Level Output File Name	: lcd

Output Format	: NGC

Optimization Goal	: Speed

Keep Hierarchy	: No

Design Statistics

# IOs	: 12

Cell Usage :

# BELS	: 30

#	GND	: 1

#	INV	: 1

#	LUT2_L	: 2

#	LUT3	: 1

#	LUT3_D	: 1

#	LUT3_L	: 3

#	LUT4	: 17

#	LUT4_L	: 3

#	VCC	: 1

# FlipFlops/Latches	: 34

#	FDC	: 22

#	FDCE	: 1

#	FDE	: 10

#	FDP	: 1

# Clock Buffers	: 1

#	BUFGP	: 1

# IO Buffers	: 11

#	IBUF	: 1

#	OBUF	: 10

=========================================================================

Device utilization summary:

Selected Device : 3s250epq208-5

| Number | of | Slices: | 24 | out | of | 2448 | 0% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number | of | Slice Flip Flops: | 34 | out | of | 4896 | 0% |
| Number | of | 4 input LUTs: | 28 | out | of | 4896 | 0% |
| Number | of | IOs: | 12 |  |  |  |  |
| Number | of | bonded IOBs: | 12 | out | of | 158 | 7% |
| Number | of | GCLKs: | 1 | out | of | 24 | 4% |