EXPERIMENT NO.4

- VHDL CODE:

-- Experiment No 4 – Mod-N Counter

-- Name : Ritesh Machindra Todmal

-- Roll No. : 404C001

-- Teacher Name: Dr.D.G.Ganage

-- Batch : C1

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.std_logic_unsigned.all;

entity counter is

port ( CLK : in std_logic;

RST : in std_logic; -- Synchronous reset input RST active

high

end counter;

Q : out std_logic_vector(3 downto 0));

architecture counter_arch of counter is signal TEMP_Q : std_logic_vector(3 downto 0); begin

process(CLK) begin

if rising_edge(CLK) then -- if clk'event and clk = '1' then

if RST = '1' then -- active high and it is inside clock therfore its synchronous reset

TEMP_Q <= (others => '0'); -- TEMP_Q <= "0000"

else

if (TEMP_Q = "1001") then

TEMP_Q <= (others => '0'); -- TEMP_Q <= "0000"

else

TEMP_Q <= TEMP_Q + 1; -- up counting end if ;

end if ; end if;

end process; Q <= TEMP_Q;

end counter_arch;

- VHDL TESTBENCH:

-- Experiment No 4 – Mod N Counter

-- Name : Ritesh Machindra Todmal

-- Roll No. : 404C001

-- Teacher Name: Dr.D.G.Ganage

-- Batch : C1

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

ENTITY counter_tb IS END counter_tb;

ARCHITECTURE behavior OF counter_tb IS

-- Component Declaration for the Unit Under Test (UUT) COMPONENT counter

PORT(

CLK : IN std_logic; RST : IN std_logic;

Q : OUT std_logic_vector(3 downto 0)

);

END COMPONENT;

--Inputs

signal CLK : std_logic := '0'; signal RST : std_logic := '0';

--Outputs

signal Q : std_logic_vector(3 downto 0);

-- Clock period definitions BEGIN

-- Instantiate the Unit Under Test (UUT) uut: counter PORT MAP (

CLK => CLK, RST => RST, Q => Q

);

-- Clock process definitions CLK_process :process

begin

end process;

CLK <= '0';

wait for 5 ns; CLK <= '1';

wait for 5 ns;

-- Stimulus process stim_proc: process begin

-- hold reset state for 100 ns.

| RST | <= | '1'; |  |  |
| --- | --- | --- | --- | --- |
|  |  | wait | for | 20 ns; |
| RST | <= | '0'; |  |  |
|  |  | wait | for | 200 ns; |
| RST | <= | '1'; |  |  |
|  |  | wait | for | 10 ns; |
| RST | <= | '0'; |  |  |
|  |  | wait | for | 300 ns; |

end process; END

- SIMULATION RESULT

- SCHEMATICS (RTL Technology)

=========================================================================

*	Final Report	*

=========================================================================

Final Results

RTL Top Level Output File Name	: counter.ngr Top Level Output File Name	: counter

Output Format	: NGC

Optimization Goal	: Speed

Keep Hierarchy	: No

Design Statistics

# IOs	: 6

Cell Usage :

# BELS	: 6

#	INV	: 1

#	LUT2	: 1

#	LUT2_L	: 1

#	LUT3	: 1

#	LUT4	: 2

# FlipFlops/Latches	: 4

#	FDR	: 4

# Clock Buffers	: 1

#	BUFGP	: 1

# IO Buffers	: 5

#	IBUF	: 1

#	OBUF	: 4

=========================================================================

Device utilization summary:

Selected Device : 3s250epq208-5

| Number | of | Slices: | 3 | out | of | 2448 | 0% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number | of | Slice Flip Flops: | 4 | out | of | 4896 | 0% |
| Number | of | 4 input LUTs: | 6 | out | of | 4896 | 0% |
| Number | of | IOs: | 6 |  |  |  |  |
| Number | of | bonded IOBs: | 6 | out | of | 158 | 3% |
| Number | of | GCLKs: | 1 | out | of | 24 | 4% |